Tomography time reduction thanks to low-level process control

Several optimization possibilities were investigated and tested in order to reduce the time needed to acquire radiographs needed for tomographic reconstruction. Low-level electrical TTL signals were employed for synchronization of the data acquisition which remedied the software communication overhead. We measured time needed to acquire a tomographic data set consisting of 2000 projections using Micos UPR 160 F Air rotary stage, PCO.4000 camera and standard software infrastructure. The rotary stage was then exchanged to AeroTech ABRT-150 AS, which has a low communication overhead and can be controlled via an ASCII interface over a TCP/IP network connection. Furthermore, low level TTL signals were employed to synchronize motor movement and image readout from the camera in such a way, that while the motor is moving, the camera is reading out the image and when the next motor position is reached, the camera sensor is exposed again. Thanks to this technique we managed to acquire the same amount of projections in only 36.3 % of the time with communication overhead and non-parallel readout. Moreover, the TTL signals were used to log motor positions in order to provide more input for the reconstruction algorithms.