Vision on WP3: Data processing evaluation and visualization

University of Civil Aviation (UCA)

Coordinator: Anton Myagotin (anton.myagotin@gmail.com; +7 (812) 704-1559)

1. Skills overview of UCA scientific development team

Programming:

Linix/GCC-based

Pure objected oriented programming with C++/STL

OpenMP / MPICH / SIMD (not ASM, GCC instincts only)  instruction sets for parallel computations

QT4 for GU interfaces

Math:

Discrete and continuous signal/image treatment, Fourier Transform

UFO-related algorithm development:

Preprocessing of radiographic and volume images –

enhancement, noise reduction, segmentation

Analytic reconstruction algorithms –

filtered-backprojection (FBP), direct Fourier inversion (DFI)

Algebraic reconstruction algorithms –basics only

Completed projects:

Efficient 3D reconstruction algorithms for computed laminography with parallel beam geometry // Joint scientific project between UCA and ISS Nr. 320/20391375/ISS (2008)

2. General remarks on WP3 documentation

Although people from BMBF-found were impressed with our proposal, in our opinion the current state of the paper is not sufficient to start a real technical discussion with who and what will do in the UFO project. For each work-package we should defined in a more exact form for the corresponding deliveries. With this small report we would like to collect our thoughts and ideas which may help to fill in and to point out those gaps.

3. Remarks on WP3.1

According to the proposal the major deliveries in this package are GPU (combined with FPGA?) computing platform and corresponding “management” software (3-layered framework).

Hardware:

Since the IPE team are the most knowledgeable in the field it is a good idea to made them a proposal for a proper hardware choice. The corresponding proposal should clear up the following questions:

What are the parts and interconnections of the computation systems?

What is a rough design for the computing platform architecture?

How many GPU cards are planned to be used? How to make the system extensible?

What video-adapters are the most suitable to perform fast parallel computations?

What is the worst case for the numerical error produced during GPU/FPGA computations? (well known fact that GPU computations suffer from imprecision, though are they acceptable to be used in reconstruction algorithms?)

What is about CPU? Is it totally excluded from the computational loop?

3-layered software framework:

Analyzing the info from the proposal we got the following framework structure.

The lowest level – a set of functions to work with hardware units (is build on the top of hardware drivers). This level includes

- functions used to read/write a small chunk of a data from/to hardware (CPU, GPU, FPGA units?)

- functions to manipulate a device state (e.g. stop/execute a computational loop)

- functions to program the unit for a particular task

At this level all computational units are visible as “individuals” each having its own peculiarities and capabilities.

Middle level – a set of functions to work with computational units

- functions for the scheduling and loading balance of the computational tasks between the different computational units.

- functions to split large data block into  smaller chunk and put down to the lowest level. Vise versa, functions for collection of small chucks into larger data blocks and send them up to the top level.

This is a higher level of abstraction. There is no possibility to distinguish between individual computational units. All of them provide the same set of control/computation functions.

Top level – a set of functions to perform trivial but efficient parallel computations (e.g. vector addition/mutiplication). At this level we have full-functioning data containers - 1D/2D/3D Array (Image). Hardware is viewed as a single computational processor (black box) with a handy set of instructions for parallel computations.

Task distribution: According to the described above structure for each level we should define interfaces (functions associated with each level) and a set of data structures (containers). Since the computational system is aimed to be used for the fast reconstructions it makes sense first to clarify top level functions (UCA together with IPE), and then go the structure down to the lowest level.

4. Remarks on WP3.2

Small remark: all hardware related information in this section does not make a lot of sense since we have already decided to “hide” hardware units behind the 3L framework in  WP 3.1. At this point we have a set of top level routines providing a set of elementary parallel computations (e.g. vector addition/multiplications). Is it right? From the same reason it is not clear what was meant with “algorithms need to be developed to automatically monitor the complete data chain from the mechanics and detector to the processing”.

The main goal to be achieved here is the implementation of reconstruction algorithms. A typical reconstruction cycle consists of the following steps:

I. Radiograph image recording

II. Radiograph image preprocessing (flat-field correction, - log(), noise reduction)

III. Reconstruction loop (e.g. in FBP we have 2 substeps: projection filtering and backprojections)

III. Volume post-processing

Assuming we have already equipment for the recording of radiographic images in a fast and robust manner, before the reconstruction loop (step III.) start the projections must be preprocessed properly (step II.). Sure this task is not so challenging as the reconstruction itself, still it requires a certain amount of work, and it is strange that there is no explicit deliveries for the preprocessing.

Before work can actually start the UFO management should decide  which reco-algorithms must be implemented: FBP, DFI, ART? Which one? All of them?  Only after the answer is clarified we can formulate a required set of parallel primitives mentioned in WP3.1.

The proposal distinguishes between laminographic and tomographic reconstruction. From a mathematical point of view the former is the general case of the latter one. Though, should we really implement two separate reconstruction routines?

Somebody should also define what filtering scheme should be implemented: Ram-Lack, Schep-Logan, Weighted window? All of them?

As it is shown above the reconstruction without radiographs preprocessing does not make much sense. The produced volume will be of a low quality. Though what preprocessing routines do we want to have?

Our proposal for the above question would be

filtering-back projection is the algorithm to be implemented first

consider lamino-reconstruction case only

(although the tomo-case can be implemented more efficiently)

use modified Ram-Lack as a filter function ([56] in the proposal)

implement basic preprocessing routines required to perform the reconstruction:

flat-filed correction and -log()

Other staff can be added later when we see the first successful reconstruction.

State of the art: Our team already implemented once the reco-algorithms in C++. Thus the major problem is to port (actually rewrite) them using the top routines of the 3L-framework.

Task distribution: According to the above scheme together with the IEKP-group we may work more on the third step (reconstruction algorithms) while the image preprocessing can be implemented by TPU groups.

5. Remarks on WP3.3

The work in the frame of this package can be split into two large problems:

- graphical user interface to adjust an experimental setup (e.g. video-camera)

- 3D volume visualization

Relating to the first question we need to consult with a beam-line scientist in order to formulate a complete and sufficient set of procedures required to manipulate an experiment setup. It should be also clarified which actual command line utilities (tools) are used to control the equipment (spec?).

For the second question we have already made some work. Currently there is a simple GUI (QT based) allowing for visualization (so far small) data sets given in EDF format. Still the management of our UFO-group should formulate (accept) exact requirements for the software, eg:

- What is the smallest allowed FPS?

- What is the largest 3D volume to be loaded?

- Which data processing tools should be implemented? (histogram, color offset, cutting, etc)

- Which data format must understand the software (EDF, RAW, …)?

Fig. 1. 3D volume rendition (Al foam) with the developed tool

Task distribution: Control GUI – TPU, Volume visualization – UCA.

Conclusion

To summarize, after the discussion is finished we have to prepare a real technical paper with an appropriate level of details for each WP.

For the task distribution the UCA-team offers the following.

WP 3.1: we start a discussion with IPE-team to clarify details of the hardware to be used and interfaces to be implemented in the 3L framework. The highest priority task is to define the top level interface of the 3L framework, since it contributes not only work in WP3.1 but also in 3.2. The SCI group helps with the implementation of the routines.

WP 3.2: Image preprocessing TPU, Reconstruction itself - IEKP & UCA.

WP 3.3: We propose the TPU-team work on the controlling staff while we would like be responsible for visualization part.